# GENERAL DESCRIPTION

The SS0804A is a CMOS LSI chip designed to drive a standard 3.5 digit biplexed LCD for a count-up/down timer in watch applications. The maximum count-down time is 20 hours with an accuracy of Seconds. The maximum count-up period is 20 hours. When the count-up time is more than 20 minutes the LCD display is in hours and minutes, seconds can be seen by pressing S2 and will revert to previous mode by releasing S2.

In the count-up/down mode, the real time can be seen by pressing switch S1 and return to the count-up/down mode by releasing S1.

Once the count-down is finished, the timer can either automatically recycle the preset time or manually reset the desired time depending on the selected bounding-pad option. The forecast alarm generates 4 alarm sounds at 10 minutes before set time. The SS0804A can be used as a parking timer, an alarm timer, a pill box timer, a kitchen timer, a sports timer, etc.

### 2	FEATURES

- A real time watch

- A 20-hour adjustable range setting count-down timer

- 20-hours of count-up time maximum

- Auto change display mode to hours and minutes when the count-up time is more than 20 minutes.

- Auto recycle or manual reset the count-down timer by bonding  option

- Hours and minutes are set independently

- Watch, Timer and Counters work independently

- Real time display in timer mode

- 5-minute and 10-minute pre-alarm function

- DC output and piezo output

- Internal voltage doubler

- Busy flag output high in timer counting

- 3.5 digit LCD display

- 32,768Hz crystal oscillator

- Single 1.5V battery operation

- ABSOLUTE MAXIMUM RATINGS ( Ta = 25oC )

| Characteristic | ## Symbol | Value | Unit |
| --- | --- | --- | --- |
| Supply Voltage | VCC | -0.3 ~ 5.0 | V |
| Input Voltage | VIN | VSS–0.3 to VCC+0.3 | V |
| Operating Temperature | TA | -20 ~ + 70 | oC |
| Storage Temperature | Tstg | - 55 ~ + 125 | oC |

- ELECTRICAL CHARACTERISTICS

(Ta = 25oC, VSS = 0V, VDD = 1.5V; unless otherwise specified)

| Characteristic | Symbol | Test Condition | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | VCC |  | 1.25 | 1.50 | 1.70 | V |
| Supply Current | ICC | No load |  |  | 2 | A |
| Display Voltage | VDD |  | 2.4 | 3.0 | 3.4 | V |
| Switch Input Current | IIN | VIN = VCC | 5 |  | 50 | A |
| Oscillator Start Voltage | VOSC | Within 3 secs | 1.35 |  |  | V |
| Oscillator Frequency | FOSC |  |  | 32,768 |  | Hz |
| Frequency Stability | Δf/f | VCC = 1.25V~1.7V |  |  | 10 | ppm |
| Output Drive Current (BD, BDN, BUSY Outputs) | IOH | VOH = 1.35V | -500 |  |  | µA |
| Alarm Output Frequency | FOUT |  |  | 4096 |  | Hz |

- OUTPUT WAVEFORM

Sends out this signal 10 minutes before the set time is up.			Sends out this signal 5 minutes before the set time is up.

BD output signal will be as this when the	BD output signal will be as this when the longterm.

count-down reach to zero.	counters reach to zero

Timer BUSY flag output.

# FUNCTIONAL DESCRIPTION

When power is turned on, all the LCD segments are illuminated to display “18:88” and the alarm sounds 2 seconds. Then the circuit automatically moves to the Real Time Setting Mode.

Time is set by the S1 for Hour and S2 for Minute setting. Colon will be on and freeze during the time setting. Depressing S3 will move the circuit to the standard Real Time Mode and Colon will begin to flash (0.5 sec on, 0.5 sec off).

If in Real Time Mode :

- key S3 is depressed more than 5 sec, the circuit will moved to the setting state again.

- press key S1 or S2, the circuit will move to the Timer/Counter Modes.

- depressing S3 will start Count-Up function.

- depressing S1 and S2 will set hour and minute for the Times and then Times will start by key S3.

In the count-down mode, when the time has gone past the set time and the alarm is stopped, the set time will remain at its current state or be reset to 0:00 depending on the bonding of the REPEAT pin to VCC.

In the timer mode, while the timer is counting, the actual time can be displayed by pressing S1. Once S1 is released, the count-down time will be shown instead.

In the count-up mode, the LCD display is normally in minutes and seconds when the count-up time is less than 20 minutes. If S2 is pressed, the LCD display will revert to hours and minutes and will return to minutes and seconds by releasing S2. The LCD display is normally in hours and minutes when the count-up time is more than 20 minutes. If S2 is pressed, the LCD display will revert to seconds and will return to hours and minutes by releasing S2.

The timer alarm will sound during the count-down process as listed below:

- 4 sounds are generated at 10 minutes before the set time is up and 8 sounds are generated at 5 minutes before the set time is up.

- Once the set time is reached, the alarm will sound 60 seconds unless it is stopped by users (by pressing S3), in which case the preset time of the timer will remain unchanged or at 0:00 depending on the status of the option pin.

- Pressing any one of the inputs (S1 or S2 or S3) will stop the alarm during the period for which the alarm is sounding.

- At 0:00 in the timer mode, the alarm will sound when S1 and S2 are simultaneously depressed. This is for the alarm test only.

- In Timer mode the LCD will display special icon: “Timer Up”, “Timer Down”.

- In the count-up/down mode the icon “colon” is flashing with frequency 2Hz when timer is counting. The icon “colon” is only displaying when timer is stopped by pressing S3.

- OPERATIONAL FLOW CHART

- LCD FORMAT

- APPLICATION CIRCUIT

SS0804A

32

31

30

29

28

27

25

26

24

23

33

22

21

20

6

5

4

3

7

8

9

10

11

12

13

14

15

16

17

18

19

2

1

S1

CAP2

CAP1

V

DD

COM1

COM2

SEGMENTS

OSCI

OSCO

BUSY

V

SS

V

CC

REPEAT

BD

BDN

S1

S3

S3

S2

S2

0.01



0.01



X-tal

32,768Hz

+

-

1.5V

PIEZO

BUZZER

1.5k



- PAD LAYOUT

SS0804A

32

31

30

29

28

27

25

26

24

23

33

22

21

20

6

5

4

3

7

8

9

10

11

12

13

14

15

16

17

18

19

2

1

(0,0)

Chip Size : 2670 x 2590

Pad Size : 120 x 120

Unit : µm

- PAD LOCATION

| Pad No. | Pad Name | X | Y | Description | Pad No. | Pad Name | X | Y | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | HSET(S1) | -1175 | 685 | Input for hour setting | 18 | K/P | 995 | -1135 | Segment drive |
| 2 | CAP2 | -1175 | 195 | Voltage doubling capacitor | 19 | NC | 1175 | -1135 |  |
| 3 | CAP1 | -1175 | -190 | Voltage doubling capacitor | 20 | COM2 | 1175 | -785 | Common 2 drive |
| 4 | VDD | -1175 | -565 | Display Supply | 21 | OSCI | 1175 | -565 | Oscillator input |
| 5 | COM1 | -1175 | -775 | Common 1 drive | 22 | OSCO | 1175 | -325 | Oscillator output |
| 6 | UP/D3 | -1175 | -1135 | Segment drive | 23 | BUSY | 1175 | 915 | Timer busy flag, active high |
| 7 | B3/C3 | -985 | -1135 | Segment drive | 24 | T2 | 1175 | 1135 | Test pin |
| 8 | A3/G3 | -805 | -1135 | Segment drive | 25 | T1 | 925 | 1135 | Test pin |
| 9 | F3/E3 | -625 | -1135 | Segment drive | 26 | VSS | 695 | 1135 | Negative power supply (GND) |
| 10 | DOWN/D2 | -445 | -1135 | Segment drive | 27 | VCC | 465 | 1135 | Positive power supply |
| 11 | B2/C2 | -265 | -1135 | Segment drive | 28 | REPEAT | 235 | 1135 | Option pin for countdown recycle |
| 12 | A2/G2 | -85 | -1135 | Segment drive | 29 | BD | -55 | 1135 | Piezo drive |
| 13 | F2/E2 | 95 | -1135 | Segment drive | 30 | BDN | -895 | 1135 | Piezo drive |
| 14 | COL/D1 | 275 | -1135 | Segment drive | 31 | NC | -1175 | 1135 | No Connection |
| 15 | B1/C1 | 455 | -1135 | Segment drive | 32 | START(S3) | -1175 | 875 | Start/Stop Input |
| 16 | A1/G1 | 635 | -1135 | Segment drive | 33 | MSET(S2) | -1175 | 435 | Input for minute setting |
| 17 | F1/E1 | 815 | -1135 | Segment drive |  |  |  |  |  |

欢迎索取免费详细资料、设计选型指南和光盘、样品；产品繁多未能尽录，欢迎来电查询。

中国传感器科技信息网：HTTP：//WWW.SENSOR-IC.COM/

工控安防网：HTTP：//WWW.PC-PS.NET/

消费电子专用电路网：HTTP://WWW.SUNSTARE.COM/

E-MAIL： xjr5@163.com  szss20@163.com

MSN: suns8888@hotmail.com

QQ:195847376

地址： 深圳市福田区福华路福庆街鸿图大厦1602室

电话：0755-83778810 83394033 83376282 83387030  83387016

传真：0755-83376182  83338339   邮编：518033  手机：(0)13902971329

深圳展销部：深圳华强北路赛格电子市场2583号  TEL/FAX：0755-83665529  13823648918 FAX:0755-82914956

北京分公司：北京海淀区知春路132号中发电子大厦3097号

TEL：010-81159046  82615020  13823791822  FAX：010-62543996

上海分公司：上海市北京东路668号上海賽格电子市场地下一层D25号

TEL：021-28311762  56703037  13823676822  FAX：021-56703037

西安分公司：西安高新开发区20所(中国电子科技集团导航技术研究所)

西安劳动南路88号电子商城二楼D23号

TEL：029-81022619  13072977981  FAX:029-88789382

成都：TEL:(0)13717066236
技术支持：0755-83394033 13501568376